Pre-Trip Vehicle Inspection: Lights

Virtual Reality Training

Practice/Evaluation Answer Key

Part 1 - Left Turn Signal

- Teleport to the cab door. Activate the door handle with your virtual hand to enter the cab.

- Grab the turn signal lever with your virtual hand and trigger button. Lower your hand and release the trigger once the turn signal activates.

- Grab the cab door handle with your virtual hand to exit the cab.

- Teleport to the front left of the tractor.

- Activate the tractor’s front left turn signal light with your laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Left Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back left of the tractor.

- Point the laser toward the tractor’s back left turn signal light. Then pull the trigger on the controller to inspect it.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Left Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back left of the trailer.

- Point the laser toward the trailer’s back left turn signal light. Then pull the trigger on the controller to inspect it.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Left Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

Part 2 - Right Turn Signal

- Teleport to the cab door. Activate the door handle with your virtual hand to enter the cab.

- Grab the turn signal lever with your virtual hand and trigger button. Move your hand up and release the trigger once the turn signal activates.

- Grab the cab door handle with your virtual hand to exit the cab.

- Teleport to the front right of the tractor.

- Point the laser toward the tractor’s front right turn signal light. Then pull the trigger on the controller to inspect it.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Right Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back right of the tractor.

- Point the laser toward the tractor’s back right turn signal light. Then pull the trigger on the controller to inspect it.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Right Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back right of the trailer.

- Point the laser toward the trailer’s back right turn signal light. Then pull the trigger on the controller to inspect it.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Right Turn Signal

  - Answer Question 2: Is the light working properly?

    - If the light is blinking on and off, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the amber housing around the bulb is intact, the correct answer is "Yes."

    - If the amber housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

Part 3 – High Beams

- Teleport to the cab door. Activate the door handle with your virtual hand to enter the cab.

- Grab the turn signal lever with your virtual hand and trigger button. Push the lever away from you and release the trigger once the high beams activate.

- Grab the cab door handle with your virtual hand to exit the cab.

- Teleport to the front left of the tractor.

- Select the left high beam with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: High Beam

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the high beam aimed properly?

    - If the light is shining on the avatar’s mid-section, the answer is "Yes."

    - If the light is shining on the avatar's legs, the answer is "No, the beam is aimed too low."

    - If the light is shining on the avatar's upper torso and head, the answer is "No, the beam is aimed too high."

  - Answer Question 4: Is the lamp the correct color?

    - If the bulb is white, the correct answer is "Yes."

    - If the bulb is blue, the correct answer is "No. Report the defect."

- Teleport to the front right of the tractor.

- Select the right high beam with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: High Beam

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the high beam aimed properly?

    - If the light is shining on the avatar’s mid-section, the answer is "Yes."

    - If the light is shining on the avatar's legs, the answer is "No, the beam is aimed too low."

    - If the light is shining on the avatar's upper torso and head, the answer is "No, the beam is aimed too high."

  - Answer Question 4: Is the lamp the correct color?

    - If the bulb is white, the correct answer is "Yes."

    - If the bulb is blue, the correct answer is "No. Report the defect."

Part 4 – Brake Lights

- Teleport to the cab door. Activate the door handle with your virtual hand to enter the cab.

- Grab the johnny bar on the dashboard with your virtual hand and trigger, and pull it down.

- Grab the cab door handle with your virtual hand to exit the cab.

- Teleport to the back left of the tractor.

- Select the tractor’s back left brake light with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Left Brake Light

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the red housing around the bulb is intact, the correct answer is "Yes."

    - If the red housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back left of the trailer.

- Selection the trailer’s back left brake light with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Left Brake Light

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the red housing around the bulb is intact, the correct answer is "Yes."

    - If the red housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back right of the trailer.

- Select the trailer’s back right brake light with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Right Brake Light

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the red housing around the bulb is intact, the correct answer is "Yes."

    - If the red housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the back right of the tractor.

- Select the tractor’s back right brake light with the laser pointer.

  - Answer Question 1: What type of light are you inspecting?

    - Correct answer: Right Brake Light

  - Answer Question 2: Is the light working properly?

    - If the light is on, the correct answer is "Yes."

    - If the light is off, the correct answer is "No. Report the defect."

  - Answer Question 3: Is the lamp the correct color?

    - If the red housing around the bulb is intact, the correct answer is "Yes."

    - If the red housing around the bulb is broken or missing, the correct answer is "No. Report the defect."

- Teleport to the Finish Inspection screen, and select "Finish Inspection" with the laser pointer.